M.I.T. LAB Assignment – 10

U19CS012

1. Program to add two numbers.

(16 bit addition, 32 bit addition, 32 bit addition using DD directives)

TASM Code [16 Bit Addition]:

model small ; code & Data Segment fits in 64 KB

.8086

.data   ; DATA SEGMENT

; Intialise the First Number [QW - (Define Word)]

num1 DW 1234H

; Intialise the Second Number

num2 DW 5140H

; Variable ('ans') to Store the Answer

ans DW ?

; Declare carry as an 8-bit variable with a value 0

carry DB 00H

.code   ; CODE SEGMENT

MOV AX, @DATA

MOV DS, AX

; Move the First Number (num1 data-word) in 'ax' reg

MOV AX, num1

; Move the Second Number (num2 data-word) in 'bx' reg

MOV BX, num2

; 16 Bit Addition

ADD AX, BX

JNC skip

MOV carry, 01H

; Store the Result in 'ans' data-word

skip: MOV ans, AX

; HLT

; 4CH is the parameter for the terminate function

MOV AH, 4CH

; If AL is 00h then the program terminated without an error.

MOV AL, 00H

INT 21h

END

Input:

Number1: (1234)H

Number2: (5140)H

Answer: (6374)H

Output:

[Due to Little Endian Rule: Higher Address -> Higher Byte & Lower Address Lower Byte]

TASM Code [32 Bit Addition]:

model small

.8086

.data   ; DATA SEGMENT

; First 32 Bit Number

num1high dw 1254H

num1low dw 25A0H

; Second 32 Bit Number

num2high dw 5835H

num2low dw 3627H

; 'ans' to Store Result of 32 Bit Addition

anshigh dw ?

anslow dw ?

; Carry

carry db 00H

.code    ; CODE SEGMENT

mov ax,@data

mov ds,ax

; Add Lower 16 Bit Part

mov ax,num1low

mov bx,num2low

add ax,bx

; Store Answer in 'anslow'

mov anslow,ax

; Add Higher 16 Bit Part

mov ax,num1high

mov bx,num2high

; Add with Carry [Generated from Addn of Lower Bit]

adc ax,bx

; Store Answer in 'anshigh'

mov anshigh,ax

; If Carry was Generated

jnc skip

mov carry, 01H

skip: mov anshigh,ax

;HLT

mov ax,4C00h

int 21h

end

Input:

Number1: (1254 25A0)H

Number2: (5835 3627)H

Answer: (6A89 5BC7)H

Output:

TASM Code [32 Bit Addition using DD Derivatives]:

model small

.8086

.data   ; DATA SEGMENT

; Double Word [DD]

num1 dd 123403A0H

num2 dd 0ABC0FFFH

; Answer & Carry

ans dd ?

carry db ?

.code   ; CODE SEGMENT

mov ax,@data

mov ds,ax

mov dl, 00H

; Intialize the Word Pointers

mov ax,word ptr num1

mov bx,word ptr num2

add ax,bx

mov word ptr ans,ax

mov ax,word ptr num1+2

mov bx,word ptr num2+2

; Add the Carry of Previous Lower 16 Bit Part

adc ax,bx

mov word ptr ans+2,ax

jnc skip

inc dl  ; Increment the Carry

skip: mov carry, dl

; HLT

mov ax,4C00h

int 21h

end

Input:

Number1: (1234 03A0)H

Number2: (OABC 0FFF)H

Answer: (1CF0 139F)H

Output:

2. Program to subtract two numbers. (16 bit subtraction, 32 bit subtraction)

TASM Code [16 Bit Subtraction]:

model small

.8086

.data   ; DATA SEGMENT

num1 dw 9876H

num2 dw 1234H

ans dw ?

borrow db 00H

.code   ; CODE SEGMENT

mov ax,@data

mov ds,ax

mov ax,num1

mov bx,num2

sub ax,bx

jnc skip

mov borrow, 01H

skip: mov ans,ax

; HLT

mov ax,4C00H

int 21h

end

Input:

Number1: (9876)H

Number2: (1234)H

Answer: (8642)H

Output:

TASM Code [32 Bit Subtraction]:

model small

.8086

.data   ; DATA SEGMENT

; Double Word [DD]

num1 dd 97DE207AH

num2 dd 46AC1313H

ans dd ?

borrow db ?

.code   ; CODE SEGMENT

mov ax,@data

mov ds,ax

mov dl,00h  ; Borrow [Intial]

mov ax,word ptr num1

mov bx,word ptr num2

sub ax,bx

mov word ptr ans,ax

mov ax,word ptr num1+2

mov bx,word ptr num2+2

; Subract with the Borrow from Previous Subtraction

sbb ax,bx

mov word ptr ans+2,ax

jnc skip

inc dl  ; Increment the Borrow

skip: mov borrow,dl

; HLT

mov ax,4C00h

int 21h

end

Input:

Number1: (97DE 207A)H

Number2: (46AC 1313)H

Answer:  (5132 0D67)H

Output:

3. Program to multiply signed 16-bit numbers

TASM Code:

model small

.8086

.data   ; DATA SEGMENT

num1 dw 0421H

num2 dw 003EH

ans dw ?

.code   ; CODE SEGMENT

mov ax,@data

mov ds,ax

mov ax,num1

imul num2

mov ans, ax

mov ax,4C00h

int 21h

end

Input:

Num1: (0421) H

Num2: (003E) H

Answer: (FFFE) H

Output:

4. Program to multiply unsigned 16-bit numbers

TASM Code:

model small

.8086

.data

num1 dw 023AH

num2 dw 0072H

ans dw ?

.code

mov ax, @data

mov ds, ax

mov ax, num1

mul num2

mov ans,ax

mov ax,4C00h

int 21h

end

Input:

Num1: (023A) H

Num2: (0072) H

Answer: (FDD4) H

Output:

5. Program for division of unsigned 8-bit numbers

TASM Code:

model small

.stack 100

.8086

.data

num1 db 9DH

; This Should not be Zero [Divide by Zero Error]

num2 db 1CH

quotient db ?

remainder db ?

.code

mov ax,@data

mov ds,ax

; Making it 16 Bit [00 num1] [(16 Bit)/(8 Bit)]

mov ah, 00H

mov al, num1

div num2

mov quotient, al

mov remainder, ah

mov ax,4C00h

int 21h

end

Input:

Num1: (9D) H

Num2: (1C) H

Answer: Quotient = (05) H & Remainder = (11)H

Output:

6. Program for division of unsigned 16-bit numbers

TASM Code:

model small

.stack 100

.8086

.data

num1 dw 2A81H

num2 dw 003CH

quotient dw ?

remainder dw ?

.code

mov ax,@data

mov ds,ax

mov ax,num1

div num2

mov quotient,ax

mov remainder,dx

mov ax,4c00h

int 21h

end

Input:

Num1: (2A81) H

Num2: (003C) H

Answer: Quotient = (B5) H & Remainder = (15)H

Output:

7. Program for division of signed 8-bit numbers

TASM Code:

.model small

.stack 100

.8086

.data

num1 db -82H

num2 db 05H

quotient dw ?

remainder dw ?

.code

mov ax, @data

mov ds, ax

mov ah, 00H

mov al, num1

; CBW (convert byte to word) [Sign Preserving]

cbw

idiv num2

mov quotient, ax

mov remainder, dx

mov ax, 4C00H

int 21h

end

Input:

Output:

8. Program for division of signed 16-bit numbers

TASM Code:

.model small

.stack 100

.8086

.data

num1 dw -4FF1H

num2 dw 0512H

quotient dw ?

remainder dw ?

.code

mov ax, @data

mov ds, ax

mov ax, num1

; CWD (convert word to doubleword)

cwd

idiv num2

mov quotient, ax

mov remainder, dx

mov ax, 4C00H

int 21h

end

Input:

Output:

SUBMITTED BY:

BHAGYA VINOD RANA

[U19CS012]